GENERAL NOTES

In the event of conflicts between this section of the documents and other documents, this section shall take precedence.

THE WORK

The work shall include but not be limited to:

- Furnish and installed the following Work at the Pre-Treatment Structure at the Water Reclamation Facility and the Electrical Building at the Master Pump Station in complete accordance with the Contract Documents.

- Pre-Treatment Structure roof is 48’-8” X 38’-8”. Installation shall be per Drawing S07-08.

  - Modified bituminous roofing system to include the following

    - Rigid Insulation: 6” flat

    - Cover board

    - Perimeter PT Nailers, as applicable

    - Flashing

    - Sealants

  - Edge coping, flashing, 6” X 6” aluminum gutter and 4” X 5” downspouts as required.

    - PT Nailers and Concrete Anchors

    - Anchor clips and fasteners as required

    - Concrete Splash Pads

- Master Pump Station Electrical Building Roof is 32’-8” X 22’-8”. Installation shall be per Drawing A-101 and A-301.

  - Modified bituminous roofing system per Specification 07525 of Volume 4 to include the following:

    - Rigid Insulation: Tapered

    - Cover board

    - Perimeter PT Nailers, as applicable

    - Flashing

    - Sealants

  - Metal Coping, 6” X 6” aluminum F shape gutter and 5” X 5” downspouts as required per Specification 07600 of Volume IV.

    - PT Nailers and Concrete Anchors

    - Anchor clips and fasteners as required

    - Concrete Splash Pads

  - Additional Items of note:

    - Subcontractor is responsible for all sealants, anchors, fasteners, etc. required for a complete installation.

    - Subcontractor is responsible for all fall protection in accordance with OSHA, this includes any anchor points that need to be installed.

    - Subcontractor is responsible for all fire protection associated with asphalt heating

    - Subcontractor is responsible for any and all temporary and night seals required to prevent water infiltration while roof is being constructed.

    - Subcontractor is responsible for a final cleaning of roof once their scope of work is complete.

    - Subcontractor is responsible to provide the Manufacturer’s Technical Field Rep for site visits/inspections as required by Specification Section 07550, 3.12.C.

    - Field tests shall be conducted by the subcontractor by a testing laboratory as required by Specification Section 07550, 3.12.B

    - Subcontractor will be responsible to pull all applicable permits as required.

SERVICES TO INCLUDE

- Submittals:

  - Shop drawings technical etc. – Furnish appropriate format and quantity of appropriate shop drawings and technical data for approval per the SCHEDULE REQUIREMENTS below.

    - Required re-submittals are due fourteen (14) calendar days after returned submittals.

  - Submit dimensional drawings, catalog data and detail drawings, and copies of all factory and performance tests as required by the specifications.

  - All certifications and reports as required.

  - Provide samples of the roofing membrane, base flashing, each type of fastener and roofing membrane seam.

- All materials must be submitted upon and approved prior to being sent to the jobsite. All non-approved items may be rejected and returned at the Subcontractor’s cost.

- Submittals shall be submitted, fully reviewed, by the Subcontractor. Each submittal shall include, up front, a copy of the specification section with each item clearly marked indicated full compliance with the specifications. Any deviations must be clearly marked and explained.

- Deliveries to be as directed by Contractor.

- All deliveries shall be made between 7am-3pm M-F. All deliveries shall be escorted onto the site by Contractor. A 72-hour notice to the Contractor’s superintendent will be required prior to any deliveries.

- Freight to the jobsites and environmental fees are included in this pricing.

- All materials supplied shall be coordinated with applicable and related specifications and drawings. The Subcontractor is responsible for reviewing and assuring that the product is correct for the application and installation.

- A schedule of values shall be provided for approval prior to billing.

SAFETY REQUIREMENTS

- Subcontractor is responsible to develop and provide a site-specific safety manual for this scope of work. Safety manual shall list all potential hazards and competent persons who will be supervising the work. This manual is required to be submitted to Contractor at least four weeks prior to starting work.

- Furnish appropriate material safety & data sheets (MSDS) and warranty documentation for approval.

- All proper PPE must be worn while onsite.

- The Subcontractor is required to abide by the Wharton-Smith, Inc. safety program and OSHA standards. Excavation, confined space plans, fall protection, and MSDS sheets will be required for all applicable work.

- Compliance with OSHA CFR 1926.1153 respirable crystalline silica standards is always required.

TECHNICAL QUALIFICATIONS STATEMENT

This Subcontract is contingent upon approval of submittals by the engineer of record in accordance with Contract Documents referenced here-in.  Any work or fabrication performed without approved submittals will be at the sole risk of the Subcontractor unless directed otherwise, by Contractor, in writing.

All work shall be performed in accordance with the project plans and specifications, Orange County Utilities Standards and Construction Specifications Manual, any/all pertinent building codes (such as any UL requirements), and OSHA regulations, without exception.

This Agreement shall be considered lump sum based on the quantities shown on the Contract Documents or quantities indicated in subsections of this agreement.

PRICING

RMS is to submit a schedule of values prior to the first pay application. The Schedule of Values must be mutually agreed upon by RMS and WSI. The pricing breakdown is as follows:

- Master Pump Station Electrical Building = $19,500.00

- Pre-Treatment Building = $38,960.00

- Additional Fire Retardant Treated Plywood and Nailer Adder:

  - MPS Electrical Building - $5,250.00

  - PTS Building - $8,640.00

- Project Total = $72,350.00